CAM Platform Selection

Recommendation: Autodesk PowerMill as Primary Integration Target

Document Type: Technical Platform Decision   |   Date: February 2026   |   Replaces: Fusion-first plan in 6-Month Roadmap v1.0

| Decision | Rationale (Summary) |
| --- | --- |
| Adopt Autodesk PowerMill as the primary CAM platform for Phase 1 integration, replacing Autodesk Fusion as the initial target. | PowerMill is the production CAM used by partner shops where we will test. Aligning our integration with the platform already on shop floors eliminates re-tooling friction and gives us real-world feedback immediately. |

# 1. Context — Why We Are Revisiting Platform Choice

The original 6-month roadmap (v1.0) identified Autodesk Fusion as the Phase 1 integration target on the basis of its Python-native CAM API and broad SME adoption. That choice was made before partner shop relationships were established. Now that we have confirmed pilot partners — shops already using PowerMill in daily production — the calculus changes substantially.

There is a fundamental principle in software product development: build for the users you have, not the users you wish you had. Our pilot shops have PowerMill running on their machines today. Every hour we spend building a Fusion plug-in is an hour we cannot use to test on their real parts. Switching the Phase 1 integration target to PowerMill is the correct decision, and this document makes the technical and strategic case for it.

◆ Governing Principle The goal of Phase 1 is to validate our AI strategy generation and physics-based cutting parameters on real production parts at real partner shops. Any integration decision that delays this validation is wrong, regardless of other technical merits. PowerMill is already on our partners' shop floors. That fact alone makes it the right Phase 1 platform.

# 2. What Autodesk PowerMill Is — Product Overview

PowerMill (officially branded as 'Autodesk Fusion with PowerMill' since 2020) is Autodesk's high-end, dedicated CAM software for complex 3- and 5-axis machining. Originally developed by Delcam (UK) under the name PowerMILL before Autodesk's 2014 acquisition, it has a 25+ year history in demanding manufacturing environments. It is the preferred platform in the mold-and-die, aerospace, automotive, and precision manufacturing industries globally.

| Original Developer | Delcam plc (Birmingham, UK) — acquired by Autodesk in 2014 for ~$230M |
| --- | --- |
| Current Branding | Autodesk Fusion with PowerMill (rebranded 2020). Still commonly referred to as 'PowerMill' by industry practitioners. |
| Target Industries | Aerospace, automotive, mold-and-die, energy, medical devices, defence |
| Machine Types | 3-axis mills, 3+2 positioning, simultaneous 5-axis machines, mill-turn, industrial robots, hybrid additive/subtractive |
| Part Types | Complex freeform surfaces, large molds and dies, turbine blades, blisks/IBRs, engine ports, manifolds, electrodes |
| Tiers | PowerMill Standard (3-axis, 3+2, basic turning, simulation) and PowerMill Ultimate (adds full 5-axis, robot offline programming, additive, blade/port/blisk strategies, electrode automation) |
| Pricing | Enterprise subscription — quote-based, typically $5,000–$15,000+ per seat per year. Not publicly listed. |
| Deployment | Windows desktop application (local install). No cloud-native CAM kernel. Post-processing outputs vendor-specific G-code / APT. |
| User Base | Tens of thousands of seats globally across aerospace, automotive, and mold/die sectors |
| Key Differentiators | Largest 5-axis strategy library of any CAM product; best-in-class surface finish quality; advanced collision avoidance; macro and template automation; proven on the most complex geometries in the world |

## 2.1 PowerMill vs. Fusion vs. NX — Where PowerMill Fits

PowerMill occupies a specific and well-defined niche in the CAM landscape. Understanding this positioning is essential for understanding why our partner shops are using it — and why it is the right platform for our pilot.

| Criterion | Autodesk Fusion | Autodesk PowerMill | Siemens NX CAM |
| --- | --- | --- | --- |
| Primary Use Case | Prototyping, SME shops, product design + manufacturing | Complex 3–5 axis milling; mold/die; aerospace; production | Aerospace/defence engineering; large enterprise integration |
| Part Complexity | Simple to moderate; solid prismatic parts | Moderate to extreme; complex freeform surfaces | Full range; tightly coupled to NX CAD model |
| 5-Axis Capability | Basic 5-axis; solid for 3+2 | Industry-leading; largest strategy library available | Strong; best when part was designed in NX |
| Surface Finish Quality | Good | Best-in-class for freeform surfaces | Very good |
| API / Automation | Python (adsk.cam); easiest to script; cloud-friendly | C# .NET + macro language; powerful but lower dev priority | NXOpen (C#/Python/Java); deep but very complex |
| AI/Automation Roadmap | Autodesk has active AI roadmap for Fusion CAM | Autodesk confirmed no current AI plans in PowerMill (Aug 2025) | Siemens + CloudNC active partnership for NX AI |
| Typical User | Product designer, SME machinist | Senior CNC programmer, production engineer | Large OEM; aerospace tier-1 supplier |
| Pricing | ~$545/year (commercial Fusion sub) | $5,000–$15,000+/seat/year (enterprise) | Similar to PowerMill; large site licenses |
| Deployment | Cloud + desktop | Desktop only (Windows) | Desktop only (Windows/Linux) |
| AI Gap (for us) | LOW — Autodesk filling this in Fusion | HIGH — confirmed empty by Autodesk; no current AI plans | MEDIUM — Siemens/CloudNC occupying this space |

★ The AI Gap in PowerMill is Confirmed and Open A community member asked Autodesk leadership directly in August 2025: 'Do you plan to apply AI in PowerMill?' The answer was explicit: 'They currently have no plans to apply AI in PowerMill. Everything you can do for automation at the moment relies on macros and plugins.' This is not a weakness to avoid — it is a market opportunity. PowerMill users are among the most demanding and highest-value CNC programmers in the world, and their platform has zero AI automation. We are building exactly that.

# 3. PowerMill Integration — Technical Architecture

PowerMill exposes three distinct integration layers. Understanding all three is critical because, unlike Fusion's clean Python API, PowerMill's automation requires combining these layers strategically. The good news: all three are well-documented, production-tested, and actively used by third-party plug-in developers.

## 3.1 Layer 1 — The Macro Language (.mac files)

PowerMill has a built-in scripting language for automation. Macros are .mac text files that execute PowerMill commands sequentially. They can create tools, set cutting parameters, create boundaries, generate toolpaths, and control virtually any aspect of the CAM session. Macros are the foundation layer — think of them as PowerMill's command line.

// Example PowerMill macro — create a tool and set parameters TOOL CREATE ; TYPE 'endmill' DIAMETER 10.0 EDIT TOOL ; OVERHANG 50.0 TOOLPATH CREATE ; TYPE 'raster_finish' EDIT TOOLPATH ; TOLERANCE 0.01 EDIT TOOLPATH ; STEPOVER 0.5 EDIT TOOLPATH ; CALCULATE

Our use of macros: the cloud AI engine generates .mac script strings that encode the full machining strategy (tool definitions, boundaries, toolpath settings, calculation commands). These scripts are then executed inside the active PowerMill session via the C# API, injecting the AI-generated strategy as native, editable PowerMill objects.

## 3.2 Layer 2 — The C# / .NET API (Primary Plug-In Interface)

The PowerShapeAndPowerMillAPI is an open-source Autodesk-maintained C# library (MIT license, hosted on GitHub) that wraps PowerMill's COM automation interface into a clean .NET object model. This is the primary API for plug-in development and external application integration.

| Library | Autodesk.PowerMill.API (NuGet package) — MIT license, actively maintained open-source |
| --- | --- |
| Language | C# (primary); VB.NET supported. Visual Studio 2015+ with .NET Framework 4.8. |
| Connection Model | COM automation — the C# application connects to a running PowerMill process via COM interop. PowerMill.dll is generated from the PowerMill executable using tlbimp.exe. |
| Key Capabilities | Create/modify toolpaths, tools, boundaries, workplanes. Execute macros. Read model geometry and existing entities. Import/export project files. Control toolpath calculation. |
| Example Operations | IMPORT TEMPLATE ENTITY TOOLPATH — import toolpath template; EDIT TOOLPATH ; CALCULATE — trigger calculation; CREATE FOLDER — organize toolpaths |
| Existing Plug-In Example | Autodesk's own addin-drilling-automation example: reads hole geometry, looks up toolpath templates in a database, and automatically assigns and calculates correct drilling toolpaths. This is a direct proof of concept for our approach. |
| Plugin Registration | COM registration via regasm.exe; loaded from PowerMill's Plugin Manager (File > Options > Manage Installed Plugins) |

// C# example — connect to running PowerMill and inject a toolpath using Autodesk.PowerMill.API; var powermill = new PowerMILLAutomation(); powermill.Connect(); // attaches to running PowerMill session // Import AI-generated toolpath template from our cloud service powermill.ExecuteEx($"IMPORT TEMPLATE ENTITY TOOLPATH '{templatePath}'"); powermill.ExecuteEx($"RENAME Toolpath # '{strategyName}'"); powermill.ExecuteEx($"EDIT TOOLPATH {strategyName} ; FEEDS SPINDLESPEED {spindleRPM}"); powermill.ExecuteEx($"EDIT TOOLPATH {strategyName} ; FEEDS CUTFEED {feedRate}"); powermill.ExecuteEx($"EDIT TOOLPATH {strategyName} ; CALCULATE");

## 3.3 Layer 3 — The PluginFramework (UI Panel)

PowerMill's PluginFramework allows third-party developers to embed a WPF (Windows Presentation Foundation) UI panel directly inside the PowerMill interface — either as a horizontal tab in the ribbon or as a docked vertical side panel. This is how we build the 'AI CAM Assistant' panel that users interact with.

| Framework File | PluginFramework40.dll — included with every PowerMill installation at <Install-Location>\file\plugins\framework\PluginFramework40\bin\Release |
| --- | --- |
| UI Technology | WPF (XAML) — standard Windows desktop UI framework. Full access to buttons, progress bars, data grids, status indicators. |
| Panel Types | HorizontalTabExample (ribbon tab) or VerticalPaneExample (side panel). Both demonstrated in Autodesk's open-source example repository. |
| Entry Point | Class implementing IPowerMILLPlugin interface. COM-registered via regasm.exe with a unique GUID. Loaded via Plugin Manager in PowerMill. |
| Our Use | Side panel showing: (1) Detected feature list with visual badges, (2) Suggested strategy with confidence score, (3) Computed feeds/speeds per operation, (4) 'Generate' and 'Refine' buttons, (5) Real-time progress indicator during cloud API call |

## 3.4 End-to-End Integration Architecture for PowerMill

The following diagram describes the complete data flow for our PowerMill plug-in, from the moment a user opens a CAD model to the moment AI-generated toolpaths appear in the PowerMill project tree:

Step 1: User opens CAD model in PowerMill (DMT / STEP / IGES / STL import) ↓ Step 2: User clicks 'Analyze' in the AI CAM Assistant side panel ↓ Step 3: C# plug-in reads active model geometry via COM API — extracts STEP/mesh reads tool library XML, machine definition, material assignment from session ↓ Step 4: Geometry + config POSTed to cloud AI engine (FastAPI / HTTPS) ↓ Step 5: Cloud runs: Feature Recognition (GNN) → Strategy Generation → Physics Engine returns JSON: [{feature, operation_type, strategy, tool, feeds, speeds, doc, stepover}] ↓ Step 6: C# plug-in displays results in side panel — user reviews and approves ↓ Step 7: Plug-in generates PowerMill macro (.mac) string from JSON response executes macro via ExecuteEx() — creates tools, boundaries, toolpaths in PM session ↓ Step 8: PowerMill calculates toolpaths (EDIT TOOLPATH ; CALCULATE) All generated toolpaths are fully editable native PowerMill objects

# 4. Known API Limitations & How We Address Them

PowerMill's automation API is powerful but has meaningful differences from Fusion's Python API. These must be understood and planned for — not discovered mid-development.

| Limitation | Impact on Us | Mitigation |
| --- | --- | --- |
| Low development priority from Autodesk. The API examples repo states 'low development priority' and is not actively maintained. | API bugs discovered after release may not be patched quickly. Features we need may be absent. | Rely only on stable, well-documented API surfaces. Supplement with macros for any unsupported operation — macros are more stable than the COM API. |
| COM-based architecture requires Windows. No cross-platform support. | Our plug-in development environment must be Windows. Cloud AI engine can be Linux, but the client-side plug-in is Windows-only. | Use Visual Studio on Windows for C# plug-in development. All ML/AI components (cloud API, model training) remain on Linux/AWS. This is standard for industrial CAM plug-ins — all major players (CloudNC, Lambda Function) follow this architecture. |
| No direct geometry query API. Cannot read B-Rep face data natively from C#. | Feature recognition cannot run client-side by interrogating the PowerMill model directly. | Export model to STEP via PowerMill export command before sending to cloud API. PowerMill can save/export STEP via macro. All geometry processing happens server-side. |
| Macro syntax is PowerMill-specific and version-sensitive. Macros that work in PM2024 may not work in PM2022. | Generated macro scripts need to be tested across PowerMill versions in use at partner shops. | Build a version detection routine in the plug-in at startup. Maintain a compatibility matrix. Test on PM2022, PM2023, PM2024, PM2025 before each release. |
| No native AI or geometry recognition in the API. Feature detection must be done externally. | We cannot call a built-in feature recognition function — we must do all AFR on our end. | This is actually an advantage: we own the AFR IP. The limitation does not block us, it defines our competitive moat. |
| Toolpath template (.ptf) import is the primary injection mechanism. Full parameter override via ExecuteEx() is limited for some advanced strategies. | Some complex 5-axis strategies may not accept all parameter overrides via macro. A pre-built library of .ptf templates may be necessary. | In Phase 1, build a library of 15–20 .ptf toolpath templates covering our core prismatic operations. Use ExecuteEx() for feeds/speeds/tolerance. Defer complex 5-axis parameter injection to Phase 3. |

# 5. Revised Phase 1 Development Plan — PowerMill Integration

The original 6-month roadmap structure (Phases 1–4 by month) remains valid. The changes required to switch from Fusion to PowerMill as the Phase 1 platform are less significant than they might appear. The AI engine, cloud API, physics models, and feature recognition architecture are entirely platform-agnostic. Only the integration layer (Layer 1 in the original architecture) changes.

✓ What Stays Identical Synthetic dataset generation (pythonOCC), AFR model training (PyTorch Geometric), physics engine (C++ mechanistic force model), cloud API (FastAPI on AWS), cutting parameter optimization (SciPy), digital twin simulation — none of these are PowerMill-specific. The same AI/ML stack built for Fusion works unchanged for PowerMill. Only the plug-in client changes.

△ What Changes The client plug-in language changes from Python to C# (.NET). The result injection mechanism changes from Fusion operation objects to PowerMill macro commands + .ptf template imports. The UI panel uses WPF/XAML instead of Fusion's JavaScript/HTML panel. Timeline impact: approximately +2 weeks to Month 1 (C# environment setup and COM binding generation) offset by -2 weeks in Month 4 (no complex Fusion API debugging). Net impact: neutral.

## 5.1 Revised Month-by-Month Timeline

| Week | Focus | Deliverable / Goal |
| --- | --- | --- |
| 1 | Infrastructure + PowerMill plug-in scaffold | C# Visual Studio project created; PowerMill.dll generated from PM2024; COM connection established; plug-in appears in PowerMill Plugin Manager; side panel visible with 'Hello World'. AWS API live with /health endpoint. |
| 2 | Model extraction + AFR training | Plug-in exports active model to STEP via macro; STEP sent to cloud API; GNN AFR model trained on 1,000 synthetic parts; /analyze endpoint returns feature JSON; feature names display in side panel. |
| 3 | Physics engine + cutting parameters | C++ mechanistic force model computing feeds/speeds for aluminum + mild steel; /parameters endpoint live; tool library import from PowerMill session (tool names, diameters, flutes); parameter output displayed in panel. |
| 4 | Macro injection + end-to-end demo | Plug-in generates macro string from AI JSON; macro executes in PowerMill session creating tools, boundaries, and toolpath stubs; EDIT TOOLPATH CALCULATE triggered; toolpaths appear in PowerMill project tree. Demo recorded. |
| 5 | Partner shop pilot | Plug-in installed at pilot shop on their PowerMill workstation; 10 real production parts processed; structured feedback collected; AFR model retrained on real parts; parameters calibrated from actual machining results. |
| 6 | Benchmark + stabilization + Phase 2 planning | Formal benchmark on 20 parts measuring programming time reduction (target ≥50%); version compatibility tested (PM2022/2023/2024/2025); documentation complete; Phase 2 spec covering NX and Mastercam written. |

## 5.2 Updated Technology Stack

| Plug-In Language | C# (.NET Framework 4.8) — required for PowerMill COM API and PluginFramework |
| --- | --- |
| Plug-In UI | WPF (XAML) using PowerMill PluginFramework40.dll — vertical side panel |
| PowerMill COM Binding | PowerMill.dll — generated from pmill.exe using tlbimp.exe (one-time setup per PM version) |
| API Library | Autodesk PowerShapeAndPowerMillAPI — MIT license, NuGet package |
| Strategy Injection | Macro string generation (.mac) + ExecuteEx() calls + .ptf toolpath template library |
| Geometry Export | STEP export via PowerMill macro command, then sent to cloud API |
| Cloud AI Engine | Unchanged — Python FastAPI on AWS, PyTorch Geometric AFR, C++ physics engine |
| IDE | Visual Studio 2022 (Windows) for plug-in; VS Code (Linux/Mac) for cloud services |
| Template Library | 15–20 .ptf toolpath templates covering roughing, semi-finishing, finishing, drilling for prismatic parts — stored in cloud and deployed with plug-in installer |
| Test Environment | PowerMill 2024 primary; validated against 2022 and 2023 for partner shop compatibility |

# 6. Market Positioning — Why PowerMill is the Right Market

Beyond the immediate practical benefit of matching our partner shops' software, there is a compelling strategic argument for PowerMill as a long-term platform focus.

## 6.1 The PowerMill User Is Our Highest-Value Customer

PowerMill is not cheap, and neither are the shops that use it. A company paying $10,000+ per seat per year for CAM software is a company for whom machining is a high-value, high-complexity core activity. These are the customers who will pay a meaningful price for AI-powered programming assistance — and for whom the ROI is immediate and measurable.

| Typical PowerMill Shop | 25–500 employees; machining complex aerospace, automotive, or mold/die parts; 5–20 CNC programmers; $10M–$500M revenue |
| --- | --- |
| Part Complexity | Molds, dies, turbine components, structural aerospace parts, engine components — parts where programming takes days to weeks, not hours |
| Programming Time (Current) | Complex 5-axis parts: 2–5 days per part. Even 3-axis mold work: 4–8 hours. AI assistance that cuts this by 50–70% saves tens of thousands of dollars per part per year. |
| Willingness to Pay | Shops already paying $10K+/seat/year for PowerMill. An AI add-on priced at $500–$2,000/month per seat is a fraction of their existing software cost with a clear, measurable ROI. |
| AI Maturity | Zero. Autodesk has confirmed no AI roadmap for PowerMill. Competitors (CloudNC, LimitlessCNC) focus on Fusion and NX. PowerMill is an uncontested AI opportunity. |

## 6.2 Competitive Landscape in the PowerMill Space

This is the most important competitive fact in this document: as of February 2026, there is no AI-powered CAM assistant specifically designed for PowerMill. None. CloudNC focuses on Mastercam, NX, Fusion, and GibbsCAM — not PowerMill. LimitlessCNC is NX-only. Lambda Function targets NX and Mastercam. Toolpath/CAM Accelerator is Fusion-only.

▶ Uncontested Market Opportunity Being the first AI CAM plug-in specifically designed for PowerMill, in a market where Autodesk has explicitly stated they will not build this themselves, and where PowerMill users represent some of the highest-value and most technically demanding CNC programmers in the world, is a first-mover advantage of the highest order. This is the correct Phase 1 market.

# 7. Fusion Integration — Deferred to Phase 2 (Not Abandoned)

Deprioritising Fusion for Phase 1 does not mean abandoning it. Fusion represents a different market segment (SMEs, product designers, job shops) that is equally valid and addressable in Phase 2. The original arguments for Fusion remain true — it simply makes more sense to build the Phase 1 pilot on the platform our partners actually use, and expand to Fusion once the core AI engine is proven.

| Phase 2 Fusion Timeline | Months 7–10 of Phase 2 (months 13–16 overall) |
| --- | --- |
| Effort Required | LOW — cloud AI engine is platform-agnostic. Only a new thin client plug-in (Python/adsk.cam) needs to be written. Estimated 4–6 weeks of plug-in development. |
| Fusion API Advantage | Fusion's Python adsk.cam API is cleaner and easier to work with than PowerMill's COM/C# stack. Phase 2 Fusion work will be faster than Phase 1 PowerMill work was. |
| Market Segment | Fusion targets job shops, SMEs, and product design companies — a different buyer persona from PowerMill's complex-manufacturing specialists. Dual-platform coverage addresses the full market. |
| Shared Components | 100% of the AI engine, cloud API, AFR model, and physics engine built for PowerMill is directly reusable for Fusion. Zero duplication of core IP. |

# 8. Risks Specific to PowerMill Integration

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| PowerMill API is deprecated or absorbed into Fusion CAM, breaking our integration | LOW — 3–5 year horizon | HIGH | PowerMill has a distinct enterprise user base that Autodesk will not alienate. Monitor Autodesk product announcements. Design our integration to be macro-centric (most stable layer) rather than COM-API-centric. |
| Partner shops using old PowerMill versions (pre-2020) with different API behavior | MEDIUM | MEDIUM | Conduct version audit of partner shops in Week 1 before writing any plug-in code. All development targets the oldest version in use. |
| C# developer not available on team (original plan assumed Python developer for Fusion) | MEDIUM | HIGH | Hire or contract a Windows/.NET developer with COM experience for Months 1–4. This is a narrow, well-defined skill set — not difficult to source. Alternatively, the lead engineer upskills in C# (2–3 week ramp from Python background). |
| Toolpath template (.ptf) library needs to be built from scratch — Autodesk does not provide a comprehensive public set | HIGH — confirmed | MEDIUM | Work directly with the pilot shop's senior CNC programmers in Month 1 to capture their existing PowerMill toolpath templates as .ptf files. These become our baseline template library. This is also a relationship-building activity with the pilot shop. |
| Autodesk suddenly announces an AI product for PowerMill, shrinking our window | LOW — contradicts Aug 2025 statement | HIGH | We have at least 12–18 months of confirmed open runway based on their August 2025 statement. Move fast. If they announce AI in PowerMill, our Phase 1 pilot data and customer relationships become an acquisition target. |

# 9. Summary & Formal Decision

✓ Formal Decision: Adopt Autodesk PowerMill as Phase 1 Primary Integration Target This document recommends, and the engineering team should adopt, Autodesk PowerMill (Autodesk Fusion with PowerMill) as the primary CAM platform for Phase 1 integration. This decision is driven by three factors of equal weight: (1) our pilot partner shops already use PowerMill in daily production, eliminating re-tooling friction, (2) there is no AI-powered CAM assistant for PowerMill today — a confirmed, uncontested market gap, and (3) the core AI engine, cloud API, and ML models are entirely platform-agnostic and require no rework. Only the client-side plug-in changes, from Python/Fusion to C#/PowerMill — an estimated two-week scope change with neutral overall timeline impact.

| Phase 1 Platform | Autodesk PowerMill — C# plug-in with WPF UI panel, macro injection, .ptf template library |
| --- | --- |
| Phase 2 Platform Addition | Autodesk Fusion — Python plug-in using adsk.cam API (4–6 week effort, fully reuses AI engine) |
| Phase 2+ Platform Addition | Siemens NX — NXOpen C#/Python API (highest-complexity integration; highest-value enterprise customers) |
| Core AI Engine | Unchanged from original plan — platform-agnostic Python/C++ cloud services on AWS |
| Timeline Impact | Net neutral — Week 1 C# setup (+2 weeks) offset by smoother Month 4 injection (-2 weeks vs. Fusion API limitations) |
| Key Immediate Action | Week 1: Conduct PowerMill version audit at all partner shops. Generate PowerMill.dll from their installed version. Begin C# Visual Studio project scaffold. |
| Key Relationship Action | Week 2: Work with partner shop's senior CNC programmer to export existing toolpath templates (.ptf) for the 10 most common part families they program. These seed our template library. |

Prepared: February 2026. This document supersedes the CAM Integration section of the AI CAM Assistant 6-Month Development Roadmap v1.0 with respect to platform selection. All other sections of the v1.0 roadmap remain current.